A.4.14	SMS/CBS 7-bit Bengali/Bangla Alphabet (0x14)

A.4.14.1	SMS/CBS 7-bit Bengali/Bangla Alphabet (0x14) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ঁ 0981 | ঐ 0990 | SP 0020 | 0 0030 | ব 09AC | া 09BE |  | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ং 0982 | _ 005F | ত 09A4 | 1 0031 | ভ 09AD | ি 09BF | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ঃ 0983 |  | ট 099F | 2 0032 | ম 09AE | ী 09C0 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | অ 0985 | ও 0993 | # 0023 | 3 0033 | য 09AF | ‌ু 200C,09C1 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | আ 0986 | ঔ 0994 | থ 09A5 | 4 0034 | র 09B0 | ‌ূ 200C,09C2 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ই 0987 | ক 0995 | ঠ 09A0 | 5 0035 |  | ‌ৃ 200C,09C3 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ঈ 0988 | খ 0996 | দ 09A6 | 6 0036 | ল 09B2 | ‌ৢ 200C,09E2 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | উ 0989 | গ 0997 | ড 09A1 | 7 0037 |  |  | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ঊ 098A | ঘ 0998 | ধ 09A7 | 8 0038 |  | ে 09C7 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ঋ 098B | ঙ 0999 | ঢ 09A2 | 9 0039 |  |  | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | চ 099A | * 002A | : 003A | স 09B8 | ৈ 09C8 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ঌ 098C | SS2 ---- | + 002B | ; 003B | ষ 09B7 | ৗ 09D7 | k 006B |  |
| 1 | 1 | 0 | 0 | C |  | ছ 099B | , 002C | ণ 09A3 | শ 09B6 |  | l 006C |  |
| 1 | 1 | 0 | 1 | D | CR 000D | জ 099C | ন 09A8 | প 09AA | হ 09B9 | ো 09CB | m 006D |  |
| 1 | 1 | 1 | 0 | E | এ 098F | ঝ 099D | . 002E | ফ 09AB | ় 09BC | ৌ 09CC | n 006E | ৽ 09FD |
| 1 | 1 | 1 | 1 | F |  | ঞ 099E | / 002F | ? 003F | ্‌ 09CD,200C | ্ 09CD | o 006F |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.14.2). When splitting a message text into submessages, there must be no cut right after an SS2. 09CD BENGALI SIGN VIRAMA: This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+09CD is a combining character with nominal glyph. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.14.2	SMS/CBS 7-bit Bengali/Bangla Alphabet (0x14) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ৪ 09EA | ৠ 09E0 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ৫ 09EB | ৡ 09E1 | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ৬ 09EC | ৄ 09C4 | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ৭ 09ED | ৣ 09E3 | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ৮ 09EE |  | D 0044 | T 0054 | ” 201D | ঽ 09BD |
| 0 | 1 | 0 | 1 | 5 | " 0022 | ৳ 09F3 | ৯ 09EF |  | E 0045 | U 0055 | ¤ 00A4 | ৰ 09F0 |
| 0 | 1 | 1 | 0 | 6 | € 20AC | ঀ 0980 |  |  | F 0046 | V 0056 | Ω 03A9 | ৱ 09F1 |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 |  |  | G 0047 | W 0057 | μ 03BC | ‍ু 200D,09C1 |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B | ‍্ 200D,09CD | H 0048 | X 0058 | ° 00B0 | ‍ূ 200D,09C2 |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D |  | I 0049 | Y 0059 | × 00D7 | ‍ৃ 200D,09C3 |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | ১⁄৪ 09E7,2044,09EA | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 | ‍ৢ 200D,09E2 |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ১⁄২ 09E7,2044,09E8 | ≠ 2260 | K 004B |  | ± 00B1 | ৎ 09CE |
| 1 | 1 | 0 | 0 | C | ! 0021 | ০ 09E6 | ৩⁄৪ 09E9,2044,09EA | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ১ 09E7 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB |  |
| 1 | 1 | 1 | 0 | E | - 002D | ২ 09E8 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ৩ 09E9 | \ 005C | · 00B7 | O 004F |  | ’ 2019 | ৺ 09FA |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |