A.4.16	SMS/CBS 7-bit Hindi (Devanagari) Alphabet (0x16)

A.4.16.1	SMS/CBS 7-bit Hindi (Devanagari) Alphabet (0x16) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ँ 0901 | ऐ 0910 | SP 0020 | 0 0030 | ब 092C | ा 093E | ॐ 0950 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ं 0902 | _ 005F | त 0924 | 1 0031 | भ 092D | ि 093F | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ः 0903 | ऒ 0912 | ट 091F | 2 0032 | म 092E | ी 0940 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | अ 0905 | ओ 0913 | # 0023 | 3 0033 | य 092F | ु 0941 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | आ 0906 | औ 0914 | थ 0925 | 4 0034 | र 0930 | ू 0942 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | इ 0907 | क 0915 | ठ 0920 | 5 0035 | ऱ 0931 | ृ 0943 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ई 0908 | ख 0916 | द 0926 | 6 0036 | ल 0932 | ॢ 0962 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | उ 0909 | ग 0917 | ड 0921 | 7 0037 | ळ 0933 | ॅ 0945 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ऊ 090A | घ 0918 | ध 0927 | 8 0038 | ऴ 0934 | ॆ 0946 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ऋ 090B | ङ 0919 | ढ 0922 | 9 0039 | व 0935 | े 0947 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | च 091A | * 002A | : 003A | स 0938 | ै 0948 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ऌ 090C | SS2 ---- | + 002B | ; 003B | ष 0937 | ॉ 0949 | k 006B | ॏ 094F |
| 1 | 1 | 0 | 0 | C | ऍ 090D | छ 091B | , 002C | ण 0923 | श 0936 | ॊ 094A | l 006C | ऺ 093A |
| 1 | 1 | 0 | 1 | D | CR 000D | ज 091C | न 0928 | प 092A | ह 0939 | ो 094B | m 006D | ऻ 093B |
| 1 | 1 | 1 | 0 | E | ऎ 090E | झ 091D | . 002E | फ 092B | ़ 093C | ौ 094C | n 006E | ॰ 0970 |
| 1 | 1 | 1 | 1 | F | ए 090F | ञ 091E | / 002F | ? 003F | ्‌ 094D,200C | ् 094D | o 006F | ्‍ 094D,200D |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.16.2). When splitting a message text into submessages, there must be no cut right after an SS2. 094D DEVANAGARI SIGN VIRAMA: This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+094D is a combining character with nominal glyph. In Devanagari, conjuncts are sometimes formed by using the half form of the first consonant, which also may be used final consonants. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.16.2	SMS/CBS 7-bit Hindi (Devanagari) Alphabet (0x16) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ४ 096A | ॠ 0960 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ५ 096B | ॡ 0961 | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ६ 096C | ॄ 0944 | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ७ 096D | ॣ 0963 | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ८ 096E | ॵ 0975 | D 0044 | T 0054 | ” 201D | ऽ 093D |
| 0 | 1 | 0 | 1 | 5 | " 0022 | ॱ 0971 | ९ 096F | ॳ 0973 | E 0045 | U 0055 | ¤ 00A4 | ऄ 0904 |
| 0 | 1 | 1 | 0 | 6 | € 20AC | ꣼ A8FC |  | ॴ 0974 | F 0046 | V 0056 | Ω 03A9 | ऑ 0911 |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 |  |  | G 0047 | W 0057 | μ 03BC | ॽ 097D |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B | ‍् 200D,094D | H 0048 | X 0058 | ° 00B0 | ॻ 097B |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D | ॎ 094E | I 0049 | Y 0059 | × 00D7 | ॹ 0979 |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | १⁄४ 0967,2044,096A | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 | ॸ 0978 |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | १⁄२ 0967,2044,0968 | ≠ 2260 | K 004B | ॶ 0976 | ± 00B1 | ॾ 097E |
| 1 | 1 | 0 | 0 | C | ! 0021 | ० 0966 | ३⁄४ 0969,2044,096A | [ 005B | L 004C | ॷ 0977 | ‰ 2030 | ॿ 097F |
| 1 | 1 | 0 | 1 | D | CSI 009B | १ 0967 | ⩽ 2A7D | ~ 007E | M 004D | ॲ 0972 | « 00AB | ॼ 097C |
| 1 | 1 | 1 | 0 | E | - 002D | २ 0968 | ⩾ 2A7E | ] 005D | N 004E | ॕ 0955 | » 00BB | ॺ 097A |
| 1 | 1 | 1 | 1 | F | ऩ 0929 | ३ 0969 | \ 005C | · 00B7 | O 004F |  | ’ 2019 | ꣽ A8FD |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |