A.4.1A	SMS/CBS 7-bit Punjabi (Gurmukhi) Alphabet (0x1A)

A.4.1A.1	SMS/CBS 7-bit Punjabi (Gurmukhi) Alphabet (0x1A) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ਁ 0A01 | ਐ 0A10 | SP 0020 | 0 0030 | ਬ 0A2C | ਾ 0A3E | ੵ 0A75 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ਂ 0A02 | _ 005F | ਤ 0A24 | 1 0031 | ਭ 0A2D | ਿ 0A3F | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ਃ 0A03 |  | ਟ 0A1F | 2 0032 | ਮ 0A2E | ੀ 0A40 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ਅ 0A05 | ਓ 0A13 | # 0023 | 3 0033 | ਯ 0A2F | ੁ 0A41 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | ਆ 0A06 | ਔ 0A14 | ਥ 0A25 | 4 0034 | ਰ 0A30 | ੂ 0A42 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ਇ 0A07 | ਕ 0A15 | ਠ 0A20 | 5 0035 | ੜ 0A5C |  | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ਈ 0A08 | ਖ 0A16 | ਦ 0A26 | 6 0036 | ਲ 0A32 |  | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ਉ 0A09 | ਗ 0A17 | ਡ 0A21 | 7 0037 | ਲ਼ 0A33 |  | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ਊ 0A0A | ਘ 0A18 | ਧ 0A27 | 8 0038 |  |  | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ੳ 0A73 | ਙ 0A19 | ਢ 0A22 | 9 0039 | ਵ 0A35 | ੇ 0A47 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ਚ 0A1A | * 002A | : 003A | ਸ 0A38 | ੈ 0A48 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B |  | SS2 ---- | + 002B | ; 003B |  |  | k 006B | ੰ 0A70 |
| 1 | 1 | 0 | 0 | C | ੲ 0A72 | ਛ 0A1B | , 002C | ਣ 0A23 | ਸ਼ 0A36 |  | l 006C | ੱ 0A71 |
| 1 | 1 | 0 | 1 | D | CR 000D | ਜ 0A1C | ਨ 0A28 | ਪ 0A2A | ਹ 0A39 | ੋ 0A4B | m 006D | ੑ 0A51 |
| 1 | 1 | 1 | 0 | E |  | ਝ 0A1D | . 002E | ਫ 0A2B | ਼ 0A3C | ੌ 0A4C | n 006E | ੶ 0A76 |
| 1 | 1 | 1 | 1 | F | ਏ 0A0F | ਞ 0A1E | / 002F | ? 003F | ੍‌ 0A4D,200C | ੍ 0A4D | o 006F | ੍‍ 0A4D,200D |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF.. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.1A.2). When splitting a message text into submessages, there must be no cut right after an SS2. 0A4D GURMUKHI SIGN VIRAMA: This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+0A4D is a combining character with nominal glyph. In Gurmukhi, conjuncts are formed by using a subscript form of the second consonant. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.1A.2	SMS/CBS 7-bit Punjabi (Gurmukhi) Alphabet (0x1A) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ੪ 0A6A |  | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ੫ 0A6B |  | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ੬ 0A6C |  | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ੭ 0A6D |  | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ੮ 0A6E |  | D 0044 | T 0054 | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 | " 0022 |  | ੯ 0A6F |  | E 0045 | U 0055 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 | € 20AC |  |  |  | F 0046 | V 0056 | Ω 03A9 |  |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 |  |  | G 0047 | W 0057 | μ 03BC |  |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B |  | H 0048 | X 0058 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D |  | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | ੧⁄੪ 0A67,2044,0A6A | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ੧⁄੨ 0A67,2044,0A68 | ≠ 2260 | K 004B |  | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | ! 0021 | ੦ 0A66 | ੩⁄੪ 0A69,2044,0A6A | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ੧ 0A67 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB |  |
| 1 | 1 | 1 | 0 | E | - 002D | ੨ 0A68 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ੩ 0A69 | \ 005C | · 00B7 | O 004F |  | ’ 2019 |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |