A.4.1C	SMS/CBS 7-bit Telugu Alphabet (0x1C)

A.4.1C.1	SMS/CBS 7-bit Telugu Alphabet (0x1C) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | ఀ 0C00 | ఐ 0C10 | SP 0020 | 0 0030 | బ 0C2C | ా 0C3E | ఁ 0C01 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ం 0C02 | _ 005F | త 0C24 | 1 0031 | భ 0C2D | ి 0C3F | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ః 0C03 | ఒ 0C12 | ట 0C1F | 2 0032 | మ 0C2E | ీ 0C40 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | అ 0C05 | ఓ 0C13 | # 0023 | 3 0033 | య 0C2F | ు 0C41 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | ఆ 0C06 | ఔ 0C14 | థ 0C25 | 4 0034 | ర 0C30 | ూ 0C42 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ఇ 0C07 | క 0C15 | ఠ 0C20 | 5 0035 | ఱ 0C31 | ృ 0C43 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ఈ 0C08 | ఖ 0C16 | ద 0C26 | 6 0036 | ల 0C32 | ౢ 0C62 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ఉ 0C09 | గ 0C17 | డ 0C21 | 7 0037 | ళ 0C33 | ౖ 0C56 | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ఊ 0C0A | ఘ 0C18 | ధ 0C27 | 8 0038 | ఴ 0C34 | ె 0C46 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ఋ 0C0B | ఙ 0C19 | ఢ 0C22 | 9 0039 | వ 0C35 | ే 0C47 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | చ 0C1A | * 002A | : 003A | స 0C38 | ై 0C48 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ఌ 0C0C | SS2 ---- | + 002B | ; 003B | ష 0C37 | ౕ 0C55 | k 006B |  |
| 1 | 1 | 0 | 0 | C |  | ఛ 0C1B | , 002C | ణ 0C23 | శ 0C36 | ొ 0C4A | l 006C |  |
| 1 | 1 | 0 | 1 | D | CR 000D | జ 0C1C | న 0C28 | ప 0C2A | హ 0C39 | ో 0C4B | m 006D |  |
| 1 | 1 | 1 | 0 | E | ఎ 0C0E | ఝ 0C1D | . 002E | ఫ 0C2B | ఼ 0C3C | ౌ 0C4C | n 006E | ॰ 0970 |
| 1 | 1 | 1 | 1 | F | ఏ 0C0F | ఞ 0C1E | / 002F | ? 003F | ్‌ 0C4D,200C | ్ 0C4D | o 006F |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.1C.2). When splitting a message text into submessages, there must be no cut right after an SS2. 0C4D TELUGU SIGN VIRAMA: This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+0C4D is a combining character with nominal glyph. In Telugu, conjuncts are often formed by using a subscript form of the second consonant. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.1C.2	SMS/CBS 7-bit Telugu Alphabet (0x1C) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ౪ 0C6A | ౠ 0C60 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ౫ 0C6B | ౡ 0C61 | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ౬ 0C6C | ౄ 0C44 | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ౭ 0C6D | ౣ 0C63 | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ౮ 0C6E |  | D 0044 | T 0054 | ” 201D | ఽ 0C3D |
| 0 | 1 | 0 | 1 | 5 | " 0022 |  | ౯ 0C6F |  | E 0045 | U 0055 | ¤ 00A4 | ౘ 0C58 |
| 0 | 1 | 1 | 0 | 6 | € 20AC | ౷ 0C77 |  |  | F 0046 | V 0056 | Ω 03A9 | ౙ 0C59 |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 |  |  | G 0047 | W 0057 | μ 03BC | ౚ 0C5A |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B |  | H 0048 | X 0058 | ° 00B0 | ౝ 0C5D |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | । 0964 | } 007D |  | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ॥ 0965 | ౧⁄౪ 0C67,2044,0C6A | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ౧⁄౨ 0C67,2044,0C68 | ≠ 2260 | K 004B |  | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | ! 0021 | ౦ 0C66 | ౩⁄౪ 0C69,2044,0C6A | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ౧ 0C67 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB |  |
| 1 | 1 | 1 | 0 | E | - 002D | ౨ 0C68 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ౩ 0C69 | \ 005C | · 00B7 | O 004F |  | ’ 2019 |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |