A.4.1E	SMS/CBS 7-bit Lao Alphabet (0x1E)

A.4.1E.1	SMS/CBS 7-bit Lao Alphabet (0x1E) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  | ຌ 0E8C | SP 0020 | 0 0030 | ມ 0EA1 | າ 0EB2 |  | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ໍ 0ECD | _ 005F | ບ 0E9A | 1 0031 | ຢ 0EA2 | ິ 0EB4 | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 |  | ຍ 0E8D | ປ 0E9B | 2 0032 | ຽ 0EBD | ີ 0EB5 | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ກ 0E81 | ຎ 0E8E | # 0023 | 3 0033 | ຼ 0EBC | ຸ 0EB8 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 | ຂ 0E82 | ຏ 0E8F | ຜ 0E9C | 4 0034 | ຣ 0EA3 | ູ 0EB9 | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 |  | ຐ 0E90 | ຝ 0E9D | 5 0035 | ລ 0EA5 | ຶ 0EB6 | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 | ຄ 0E84 | ຑ 0E91 | ພ 0E9E | 6 0036 | ວ 0EA7 | ື 0EB7 | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 |  | ຒ 0E92 | ຟ 0E9F | 7 0037 | ສ 0EAA |  | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 | ຆ 0E86 | ຓ 0E93 | ຠ 0EA0 | 8 0038 | ຩ 0EA9 | ເ 0EC0 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 | ງ 0E87 | ດ 0E94 | ໞ 0EDE | 9 0039 | ຨ 0EA8 | ແ 0EC1 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ຕ 0E95 | * 002A | : 003A | ຫ 0EAB | ໃ 0EC3 | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B | ຈ 0E88 | SS2 ---- | + 002B | ; 003B | ຬ 0EAC | ໄ 0EC4 | k 006B | ່ 0EC8 |
| 1 | 1 | 0 | 0 | C | ຉ 0E89 | ຖ 0E96 | , 002C |  | ອ 0EAD | ົ 0EBB | l 006C | ້ 0EC9 |
| 1 | 1 | 0 | 1 | D | CR 000D | ທ 0E97 | ໟ 0EDF | ະ 0EB0 | ຮ 0EAE | ໂ 0EC2 | m 006D | ໊ 0ECA |
| 1 | 1 | 1 | 0 | E | ຊ 0E8A | ຘ 0E98 | . 002E | ັ 0EB1 |  |  | n 006E | ໋ 0ECB |
| 1 | 1 | 1 | 1 | F |  | ນ 0E99 | / 002F | ? 003F | ຺ 0EBA | ໎ 0ECE | o 006F | ໆ 0EC6 |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). The Lao alphabet has two (interrelated) coding exceptions: 1) Some vowel characters have a “logical order exception”; 2) Some vowels that are written with several vowel characters do not have a combined character encoded. SP: In Lao, spaces are not used between words, only between phrases. LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.1E.1). When splitting a message text into submessages, there must be no cut right after an SS2. |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.1E.2	SMS/CBS 7-bit Lao Alphabet (0x1E) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ໔ 0ED4 | ຳ 0EB3 | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ໕ 0ED5 |  | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ໖ 0ED6 |  | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ໗ 0ED7 | ໜ 0EDC | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ໘ 0ED8 | ໝ 0EDD | D 0044 | T 0054 | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 | " 0022 | ฿ 0E3F | ໙ 0ED9 |  | E 0045 | U 0055 | ¤ 00A4 |  |
| 0 | 1 | 1 | 0 | 6 | € 20AC | ₭ 20AD | ๏ 0E4F |  | F 0046 | V 0056 | Ω 03A9 |  |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 | ๛ 0E5B |  | G 0047 | W 0057 | μ 03BC |  |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B |  | H 0048 | X 0058 | ° 00B0 |  |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | ຯ 0EAF | } 007D |  | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ๚ 0E5A | ໑⁄໔ 0ED1,2044,0ED4 | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ໑⁄໒ 0ED1,2044,0ED2 | ≠ 2260 | K 004B | ໌ 0ECC | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | ! 0021 | ໐ 0ED0 | ๓⁄໔ 0E53,2044,0ED4 | [ 005B | L 004C |  | ‰ 2030 |  |
| 1 | 1 | 0 | 1 | D | CSI 009B | ໑ 0ED1 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB |  |
| 1 | 1 | 1 | 0 | E | - 002D | ໒ 0ED2 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB |  |
| 1 | 1 | 1 | 1 | F |  | ໓ 0ED3 | \ 005C | · 00B7 | O 004F |  | ’ 2019 |  |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044 FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. |  |  |  |  |  |  |  |  |  |  |  |  |