A.4.20	SMS/CBS 7-bit Manipuri (Meetei Mayek) Alphabet (0x20)

A.4.20.1	SMS/CBS 7-bit Manipuri (Meetei Mayek) Alphabet (0x20) base table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 |  |  | SP 0020 | 0 0030 | ꯕ ABD5 | ꯥ ABE5 | ꫲ AAF2 | p 0070 |
| 0 | 0 | 0 | 1 | 1 | ꯪ ABEA | _ 005F | ꯇ ABC7 | 1 0031 | ꯚ ABDA | ꯤ ABE4 | a 0061 | q 0071 |
| 0 | 0 | 1 | 0 | 2 | ꫵ AAF5 | ꫡ AAE1 | ꫤ AAE4 | 2 0032 | ꯃ ABC3 | ꫫ AAEB | b 0062 | r 0072 |
| 0 | 0 | 1 | 1 | 3 | ꯑ ABD1 |  | # 0023 | 3 0033 | ꯌ ABCC | ꯨ ABE8 | c 0063 | s 0073 |
| 0 | 1 | 0 | 0 | 4 |  |  | ꯊ ABCA | 4 0034 | ꯔ ABD4 | ꫬ AAEC | d 0064 | t 0074 |
| 0 | 1 | 0 | 1 | 5 | ꯏ ABCF | ꯀ ABC0 | ꫥ AAE5 | 5 0035 |  |  | e 0065 | u 0075 |
| 0 | 1 | 1 | 0 | 6 |  | ꯈ ABC8 | ꯗ ABD7 | 6 0036 | ꯂ ABC2 |  | f 0066 | v 0076 |
| 0 | 1 | 1 | 1 | 7 | ꯎ ABCE | ꯒ ABD2 | ꫦ AAE6 | 7 0037 |  |  | g 0067 | w 0077 |
| 1 | 0 | 0 | 0 | 8 |  | ꯘ ABD8 | ꯙ ABD9 | 8 0038 |  | ꯦ ABE6 | h 0068 | x 0078 |
| 1 | 0 | 0 | 1 | 9 |  | ꯉ ABC9 | ꫧ AAE7 | 9 0039 | ꯋ ABCB | ꯩ ABE9 | i 0069 | y 0079 |
| 1 | 0 | 1 | 0 | A | LF 000A | ꯆ ABC6 | * 002A | : 003A | ꯁ ABC1 | ꫭ AAED | j 006A | z 007A |
| 1 | 0 | 1 | 1 | B |  | SS2 ---- | + 002B | ; 003B | ꫪ AAEA | ꯣ ABE3 | k 006B |  |
| 1 | 1 | 0 | 0 | C |  | ꫢ AAE2 | , 002C | ꫨ AAE8 | ꫩ AAE9 | ꯧ ABE7 | l 006C | ꯬ ABEC |
| 1 | 1 | 0 | 1 | D | CR 000D | ꯖ ABD6 | ꯅ ABC5 | ꯄ ABC4 | ꯍ ABCD | ꫮ AAEE | m 006D | ꫱ AAF1 |
| 1 | 1 | 1 | 0 | E | ꫠ AAE0 | ꯓ ABD3 | . 002E | ꯐ ABD0 |  | ꫯ AAEF | n 006E | ꫳ AAF3 |
| 1 | 1 | 1 | 1 | F |  | ꫣ AAE3 | / 002F | ? 003F | ꫶ AAF6 | ꯭ ABED | o 006F | ꫴ AAF4 |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). LF: LINE FEED; moves to the next line (with implied carriage return). CR: CARRIAGE RETURN; CR is not used as CR but is used as a filler after the actual SMS/CBS message text. CR should not occur inside a message text, but if it does, it must be converted to LF. Note though that <SS2,CR> is actually CSI (se next table), and that “CR” is not a filler, nor LF. SS2: SINGLE SHIFT TWO; This code shifts the next 7-bit code unit to refer to an extension of this table (subclause A.4.20.1). When splitting a message text into submessages, there must be no cut right after an SS2. U+ABED: MEETEI MAYEK APUN IYEK: Modern virama. In Meitei Mayek U+ABED is a combining character with nominal glyph. It is not used on final consonants; instead use the half forms (LONSUM). (There is also the traditional virama, U+AAF6, MEETEI MAYEK VIRAMA). |  |  |  |  |  |  |  |  |  |  |  |  |

A.4.20.2	SMS/CBS 7-bit Manipuri (Meetei Mayek) Alphabet (0x20) extension (SS2) table

|  |  |  |  | b7 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b6 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
|  |  |  |  | b5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| b4 | b3 | b2 | b1 | 1B | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | @ 0040 | < 003C | ꯴ ABF4 |  | ¦ 007C | P 0050 | ¼ 00BC | ⚠ 26A0 |
| 0 | 0 | 0 | 1 | 1 | £ 00A3 | = 003D | ꯵ ABF5 |  | A 0041 | Q 0051 | ½ 00BD | ∞ 221E |
| 0 | 0 | 1 | 0 | 2 | $ 0024 | > 003E | ꯶ ABF6 |  | B 0042 | R 0052 | ¾ 00BE | ² 00B2 |
| 0 | 0 | 1 | 1 | 3 | ¥ 00A5 | ₹ 20B9 | ꯷ ABF7 |  | C 0043 | S 0053 | “ 201C | ³ 00B3 |
| 0 | 1 | 0 | 0 | 4 | § 00A7 | ^ 005E | ꯸ ABF8 |  | D 0044 | T 0054 | ” 201D |  |
| 0 | 1 | 0 | 1 | 5 | " 0022 |  | ꯹ ABF9 |  | E 0045 | U 0055 | ¤ 00A4 | ꯛ ABDB |
| 0 | 1 | 1 | 0 | 6 | € 20AC |  |  |  | F 0046 | V 0056 | Ω 03A9 | ꯡ ABE1 |
| 0 | 1 | 1 | 1 | 7 | % 0025 | ( 0028 |  |  | G 0047 | W 0057 | μ 03BC | ꯠ ABE0 |
| 1 | 0 | 0 | 0 | 8 | & 0026 | ) 0029 | { 007B |  | H 0048 | X 0058 | ° 00B0 | ꯟ ABDF |
| 1 | 0 | 0 | 1 | 9 | ' 0027 | ꫰ AAF0 | } 007D |  | I 0049 | Y 0059 | × 00D7 |  |
| 1 | 0 | 1 | 0 | A | FF 000C | ꯫ ABEB | ꯱⁄꯴ ABF1,2044,ABF4 | ≈ 2248 | J 004A | Z 005A | ÷ 00F7 |  |
| 1 | 0 | 1 | 1 | B | − 2212 | SS3 ---- | ꯱⁄꯲ ABF1,2044,ABF2 | ≠ 2260 | K 004B |  | ± 00B1 |  |
| 1 | 1 | 0 | 0 | C | ! 0021 | ꯰ ABF0 | ꯳⁄꯴ ABF3,2044,ABF4 | [ 005B | L 004C |  | ‰ 2030 | ꯞ ABDE |
| 1 | 1 | 0 | 1 | D | CSI 009B | ꯱ ABF1 | ⩽ 2A7D | ~ 007E | M 004D |  | « 00AB | ꯝ ABDD |
| 1 | 1 | 1 | 0 | E | - 002D | ꯲ ABF2 | ⩾ 2A7E | ] 005D | N 004E |  | » 00BB | ꯢ ABE2 |
| 1 | 1 | 1 | 1 | F |  | ꯳ ABF3 | \ 005C | · 00B7 | O 004F |  | ’ 2019 | ꯜ ABDC |
| In the event that an MS receives a code where a symbol is not represented in the above table then the MS shall display the REPLACEMENT CHARACTER (U+FFFD). FF: FORM FEED; This code is defined as a Page Break character (with implied carriage return). Any mobile station which does not have pagination shall treat is as a LF (with implied carriage return). CSI: CONTROL SEQUENCE INTRODUCER; enables decimal character references, enables styling. An MS which does not support CSI shall convert it to REPLACEMENT CHARACTER. SS3: SINGLE SHIFT THREE; This code is reserved for the extension to another extension table. When splitting a message text into submessages, there must be no cut right after an SS3. U+2044: FRACTION SLASH; the sequence may need to be isolated from adjacent digits by ZWNJ. U+AAF6: MEETEI MAYEK VIRAMA: Traditional virama. This is a control character for forming conjuncts. If the display font does not support the requested conjunct, U+AAF6 is a combining character but without nominal glyph (but use underscript + as suggested in charts) |  |  |  |  |  |  |  |  |  |  |  |  |